MOODLENS

Real-Time Facial Expression Analysis

Project Codebase Inspection & Technical Report

Project by Amulya Gavankar for Real time mood detection

| Report scope | Summary |
| --- | --- |
| Application | Browser-based live facial-expression demo with webcam streaming and still-image analysis. |
| Backend | FastAPI REST endpoints and WebSocket stream; OpenCV-based engine interface. |
| Analysis engines | OpenCV Haar Cascade smile approximation; optional DeepFace seven-category analysis. |
| Client | HTML, CSS, JavaScript and Canvas; no frontend build system required. |
| Deployment | Dockerfile and Docker Compose configuration; environment-driven settings. |

Important interpretation note

MoodLens produces model-based facial-expression estimates. These outputs can be inaccurate and should not be treated as proof of a person’s internal emotional state or used as the sole basis for consequential decisions.

# 1. Executive Summary

MoodLens is a compact, end-to-end computer-vision application that analyzes faces from a browser webcam stream or an uploaded image. A browser client sends resized JPEG frames to a Python FastAPI service. The service decodes each frame, invokes a selected analysis engine, optionally stabilizes live detections with a face tracker, and returns structured JSON. The browser renders bounding boxes and mood indicators over the video and updates session-level visualizations.

The repository has a useful separation of concerns: HTTP/WebSocket orchestration lives in backend/app/main.py; engine implementations are in engines.py; configuration is centralized in config.py; tracking and smoothing are in tracker.py; request limiting is isolated in rate_limit.py; and the client behavior is contained in frontend/app.js. The Docker setup provides a reproducible starting point, while the test suite covers basic API, WebSocket, and tracker behavior.

## 1.1 Overall assessment

| Dimension | Assessment | Reason |
| --- | --- | --- |
| Architecture | Good prototype structure | Separate modules for routing, engines, configuration, tracking and rate limiting. |
| Core functionality | Implemented | Webcam stream, image upload, engine selection, overlays and dashboard. |
| Testing | Foundational | Tests cover representative routes and tracker behavior; not a full load/security/accuracy evaluation. |
| Security posture | Needs hardening | No built-in authentication; uploaded content and real-time traffic need defensive limits and deployment controls. |
| Model validity | Unvalidated in repository | No evidence in the inspected test suite of a formal, labeled accuracy or fairness benchmark. |
| Production readiness | Not yet | Requires security, observability, scale testing, privacy controls and documented evaluation. |

## 1.2 Key strengths

Two engine options allow a lightweight default and a richer optional analysis path.

WebSocket streaming supports interactive, low-latency feedback without repeated HTTP requests per frame.

Tracking and score smoothing are designed to reduce flicker between consecutive frames.

The project includes configuration through environment variables, Docker assets, API documentation support and tests.

## 1.3 Main recommendations

Prioritize request-size and resource limits, authentication or network-level access control, clear operational logging, and measured model evaluation. Treat the application as a demonstration until these controls and an appropriate validation study are in place.

# 2. Project Goals, Scope and User Experience

## 2.1 Purpose

The application demonstrates how a browser interface, a computer-vision backend, and a real-time transport layer can be combined into a usable facial-expression visualization tool. Its goal is interactive analysis and exploration, not clinical assessment or definitive emotion recognition.

## 2.2 Main user-facing capabilities

| Capability | Behavior |
| --- | --- |
| Live webcam | Requests camera access, captures frames, sends JPEG data through a WebSocket, and displays detected-face overlays. |
| Image upload | Accepts an image in the browser and submits it to the image-analysis endpoint for a one-off result. |
| Engine switching | Lets users select the Haar or DeepFace engine when that engine is available on the server. |
| Face overlays | Draws bounding boxes/corner brackets and visualizes the current estimated label and score information. |
| Mood dashboard | Shows mood bars, a timeline, latency/frame-rate indicators and session statistics. |
| Snapshot | Saves a browser-side image with visual overlays. |
| Session reset | Clears client-side history/statistics and sends a tracker reset message to the live session. |

## 2.3 Intended operating flow

The user opens the application in a modern browser and chooses live mode or upload mode.

For live mode, the browser requests camera access and establishes a WebSocket connection.

Frames are resized/encoded in the browser and sent as binary JPEG messages.

The backend decodes the frame, runs the selected engine and returns face boxes and score maps.

For streaming, the tracker associates detections with existing tracks and smooths values.

The browser draws overlays and updates visual indicators; the user may switch engines, reset the session or save a snapshot.

## 2.4 Scope boundaries

The repository is a single application rather than a trained-model research pipeline. It does not, by itself, establish accuracy across demographics, camera conditions, cultures or environments. The supplied project artifacts include an example DeepFace output/snapshot/video, but those examples should not be interpreted as a benchmark or a reproducible evaluation set.

# 3. System Architecture and Data Flow

## 3.1 Logical architecture

| Layer | Module / technology | Responsibility |
| --- | --- | --- |
| Presentation | frontend/index.html, style.css, app.js | Controls, camera capture, upload, canvas rendering, chart-like timeline and session state. |
| Transport/API | FastAPI in backend/app/main.py | Health/engine routes, upload analysis, WebSocket lifecycle and response formatting. |
| Image processing | OpenCV + NumPy | Decode image bytes, represent frames as arrays, resize and prepare frames for analysis. |
| Engine abstraction | backend/app/engines.py | Common face-result structure and registry for Haar and DeepFace engines. |
| Live stabilization | backend/app/tracker.py | Associate boxes across frames, assign IDs, smooth boxes/scores and expire stale tracks. |
| Configuration | backend/app/config.py | Environment-driven defaults for engine, width, smoothing, track TTL and rate limits. |
| Request protection | backend/app/rate_limit.py | In-memory sliding-window counters used to constrain request/frame frequency. |
| Packaging | Dockerfile, docker-compose.yml | Build and launch the service; optionally install DeepFace dependencies. |

## 3.2 Data flow

Input: webcam frames captured in the browser or a user-selected image file.

Encoding and transport: JPEG bytes are transmitted over WebSocket for live analysis; multipart form upload is used for still images.

Decode and validation: the backend attempts to decode the received bytes into an image array and rejects invalid still-image data.

Inference: the chosen engine returns zero or more face detections, each with a bounding box and score dictionary.

Post-processing: live detections pass through the tracker for matching and smoothing; one-off upload analysis returns direct engine results.

Output: JSON includes fields such as image dimensions, selected engine, latency, and face results; the frontend renders the output.

## 3.3 Important design choice

The browser performs capture and visualization while the server performs inference. This keeps model dependencies off the client and allows engine selection centrally. The trade-off is that image frames are transmitted to the server, so network latency, server capacity, and privacy/security controls become important.

## 3.4 State and scaling

Rate-limit counters and tracker/session state are held in process memory. This is simple for a single instance, but multiple workers or replicas can have inconsistent counters/state. A scaled deployment should use a shared rate-limit store and explicitly decide whether live sessions remain pinned to a single worker.

# 4. Backend Components and API

## 4.1 API surface

| Endpoint | Purpose | Expected behavior |
| --- | --- | --- |
| GET /api/health | Health status | Returns an OK status and configured default engine. |
| GET /api/engines | Engine discovery | Reports engine availability, categories and default selection. |
| POST /api/analyze | Still-image inference | Accepts multipart field `file`; optional `engine` query parameter; returns image metadata and face results. |
| WS /ws | Live inference | Receives binary JPEG frames and sends JSON results; text messages can change engine or reset tracking. |
| GET / | Dashboard | Serves the frontend entry page; static frontend assets are exposed by the application. |

## 4.2 Result model

The engines share a common raw-face representation containing a bounding box and a dictionary of category scores. The API formats detections for the browser, including a predicted label/confidence and score values. Live responses additionally benefit from track identifiers and smoothed estimates.

## 4.3 WebSocket controls

| Message | Purpose | Operational note |
| --- | --- | --- |
| Binary JPEG frame | Analyze the next frame | Excess frames can be skipped by the configured rate limit; malformed frames are ignored. |
| {"engine":"deepface"} | Switch analysis engine | An unsupported/unavailable engine produces an error response; tracker state is reset when changing engine. |
| {"reset":true} | Reset tracking | Clears current face tracks for the live session. |

## 4.4 Configuration variables

| Variable | Default | Meaning |
| --- | --- | --- |
| MOODLENS_ENGINE | haar | Default engine. |
| MOODLENS_MAX_WIDTH | 640 | Maximum frame width before backend-side resizing. |
| MOODLENS_ALPHA | 0.45 | Weight used for smoothing new tracker values. |
| MOODLENS_TRACK_TTL | 1.5 s | Time without a match before a track expires. |
| MOODLENS_ANALYZE_RATE_LIMIT | 30 | Upload-analysis requests allowed per client per window. |
| MOODLENS_ANALYZE_RATE_WINDOW | 60 s | Upload-analysis rate-limit window. |
| MOODLENS_WS_FRAME_RATE_LIMIT | 10 | WebSocket frames allowed per client per window. |
| MOODLENS_WS_FRAME_RATE_WINDOW | 1 s | WebSocket frame-rate window. |

Rate limits are process-local and IP-based according to the project documentation. Behind a reverse proxy, trusted client-IP handling must be configured carefully; arbitrary forwarded headers should not be trusted.

# 5. Analysis Engines and Face Tracking

## 5.1 Haar Cascade engine

The default engine uses OpenCV Haar cascades for face detection and smile detection. Its output maps smiling faces toward `happy` and non-smiling detections toward a `neutral` approximation. This is lightweight and avoids the optional DeepFace dependency, but the label is a smile-based heuristic rather than a general seven-class expression classifier.

Advantages: low dependency footprint, comparatively light deployment and a fast path for demos.

Limitations: sensitive to pose, lighting, face scale and smile visibility; cannot meaningfully distinguish the full range of expression categories.

## 5.2 Optional DeepFace engine

The optional DeepFace integration provides the seven categories documented by the project: happy, sad, angry, surprise, fear, disgust and neutral. It requires the additional requirements file and may download model data on first use. Model initialization can make the first analysis slower than subsequent requests.

Advantages: richer category output than the Haar smile heuristic.

Limitations: heavier dependencies and runtime; model outputs remain estimates and may be affected by image quality, pose, lighting and demographic variation.

The engine implementation catches analysis exceptions and can return an empty result. This protects the request path from some model errors, but it can obscure the difference between 'no face detected' and 'inference failed' unless failures are logged or surfaced distinctly.

## 5.3 Tracking and smoothing

The tracker associates new detections with existing tracks using Intersection over Union (IoU), a measure of overlap between two bounding boxes. It preserves identifiers when matching succeeds, smooths score/box updates with an exponential-style weight, and expires tracks after a configured time-to-live.

| Mechanism | Benefit | Known limitation |
| --- | --- | --- |
| IoU matching | Simple and efficient association between nearby detections. | Large movement, occlusion or crowded scenes can break matches or switch IDs. |
| Score smoothing | Reduces abrupt label/score changes from frame to frame. | Adds lag and can hide rapid genuine changes in model output. |
| Track TTL | Removes stale tracks after a face disappears. | TTL choice affects flicker and stale identity persistence. |

## 5.4 Responsible interpretation

Facial appearance is not a reliable standalone proxy for a person's internal emotional state. The interface should use language such as 'predicted facial-expression category' or 'model estimate', explain uncertainty, and avoid high-stakes use such as hiring, grading, discipline, healthcare decisions or monitoring without informed consent and rigorous justification.

# 6. Frontend, Deployment and Developer Workflow

## 6.1 Frontend implementation

The frontend is intentionally framework-free. HTML defines the dashboard, CSS provides styling, and JavaScript coordinates camera permissions, WebSocket messages, image uploads, canvas overlays, mood bars, timeline updates, session statistics and snapshots. This reduces setup complexity and removes the need for a separate JavaScript package/build step.

Live camera uses browser media APIs and sends JPEG frames at a controlled cadence.

Image upload sends multipart form data to the REST analysis endpoint.

The engine picker checks backend availability and warns when an optional engine is missing.

The UI displays latency and a frame-rate indicator; these are useful operational hints, not a full performance benchmark.

## 6.2 Local setup

Create and activate a Python virtual environment.

Install base dependencies with `pip install -r requirements.txt`.

From the repository root, start the service with `uvicorn backend.app.main:app --reload`.

Open `http://localhost:8000`; camera permissions are generally available on localhost or HTTPS.

For seven-category analysis, install `requirements-deepface.txt` and set `MOODLENS_ENGINE=deepface` or select it in the dashboard.

## 6.3 Docker setup

The Dockerfile uses a Python 3.12 slim base image, installs the base requirements, copies backend and frontend code, exposes port 8000 and defines a health check. A build argument (`DEEPFACE`) optionally installs the additional model dependencies. Docker Compose maps port 8000 and defaults to the Haar engine.

## 6.4 Operational considerations

The DeepFace image will be larger and may need additional memory, disk and startup time.

Use HTTPS for remote browser camera access and TLS for public traffic.

Set conservative worker/concurrency limits until load testing establishes safe capacity.

Use structured logs and metrics for inference latency, error rate, rate-limit events and active WebSocket sessions.

Pin and regularly review dependency versions before a production release; broad minimum-version ranges can make builds change over time.

# 7. Testing, Quality and Known Gaps

## 7.1 Existing test coverage

The repository's Pytest suite uses FastAPI TestClient and synthetic blank/invalid images. The visible tests cover:

Health endpoint and engine listing.

Serving the dashboard and JavaScript asset.

Analysis of a blank image and rejection of undecodable image bytes.

Unknown engine handling.

A WebSocket request/response round trip and invalid engine control message.

Tracker ID continuity, score smoothing and stale-track expiration.

## 7.2 What the tests do not establish

| Gap | Why it matters | Suggested validation |
| --- | --- | --- |
| Model accuracy | API correctness does not show whether expressions are classified correctly. | Use a labeled, consented evaluation set; report per-class precision/recall/F1 and confusion matrix. |
| Fairness/robustness | Performance can vary with lighting, skin tone, age, pose and camera quality. | Report subgroup and condition-level results where ethically and legally appropriate. |
| Load and concurrency | No evidence of sustained multi-client capacity. | Load-test uploads and concurrent WebSocket streams; measure p50/p95 latency, errors and memory. |
| Security | Basic functional tests do not test abuse cases. | Test size limits, malformed inputs, rate-limit behavior, origin policy and access controls. |
| Operational failures | Silent model failures may look like no detections. | Inject engine failures and verify logs, metrics and clear error responses. |

## 7.3 Codebase risks to address

Enforce maximum upload bytes before reading the entire body into memory; also validate decoded dimensions and image formats.

Set a maximum WebSocket frame size and bound per-client memory/CPU usage.

Add authentication or restrict the service to a trusted network; CORS/origin settings alone are not authentication.

Use a shared rate limiter for multi-worker deployments, or enforce limits at a trusted gateway.

Separate 'no face found' from 'engine failed' in logs and API status semantics.

Add automated dependency/security scanning and a reproducible lock/pinning strategy.

# 8. Security, Privacy and Responsible Use

## 8.1 Data protection considerations

Webcam frames and uploaded photographs may contain identifiable biometric imagery. The architecture transmits frames from the browser to the backend for analysis. The inspected repository does not establish a full privacy governance layer, so a deployment owner should explicitly document what is processed, where it is processed, whether any data is stored, who can access it, and when it is deleted.

## 8.2 Minimum safeguards before external deployment

Require authentication or place the service behind a trusted access gateway.

Use HTTPS/WSS and configure trusted proxy headers correctly.

Limit request/frame sizes, image dimensions, request rates, active connections and inference concurrency.

Avoid persisting raw frames unless strictly necessary; if retained, define consent, retention, deletion and access policies.

Log operational metadata rather than raw images or sensitive payloads; protect logs and secrets.

Provide clear user notice and consent before camera capture, and make stopping capture straightforward.

Document the intended purpose and prohibit unsupported high-stakes inference uses.

## 8.3 Threat and mitigation summary

| Risk | Potential impact | Mitigation |
| --- | --- | --- |
| Oversized/malformed image | Memory pressure, excessive decode work or service failure. | Byte/dimension limits, strict decoding and bounded error handling. |
| Unauthenticated access | Unwanted use of camera-analysis API or resource exhaustion. | Authentication, network restrictions, TLS and monitoring. |
| Distributed deployment | Rate-limit bypass across workers/replicas. | Shared rate-limit backend or gateway enforcement. |
| Silent inference exception | Misleading empty result and difficult debugging. | Structured error logs, metrics and explicit failure status. |
| Misinterpretation of labels | Unfair or consequential decisions based on unreliable estimates. | Clear caveats, restricted use, validation and human oversight. |

## 8.4 Responsible-use position

The application is best positioned as a learning/demo tool for visualizing computer-vision predictions. A production deployment should not infer personality, intent, truthfulness or mental health from facial-expression labels. Any use involving people should be consent-based, proportionate, transparent and subject to appropriate legal and ethical review.

# 9. Improvement Roadmap and Conclusion

## 9.1 Prioritized roadmap

| Priority | Work item | Acceptance criterion |
| --- | --- | --- |
| P0 — Safety | Add upload/frame byte limits, decoded dimension caps and bounded concurrency. | Oversized and malformed inputs fail safely without unbounded memory growth. |
| P0 — Access | Add authentication or trusted-network access control; require TLS in deployment. | Unauthenticated external clients cannot invoke analysis endpoints. |
| P1 — Observability | Add structured logs and metrics for engine failures, latency and rate limiting. | Operators can distinguish no-face results from inference errors. |
| P1 — Validation | Build a consented labeled test set and document evaluation methodology. | Report per-class metrics, confusion matrix and known limitations. |
| P1 — Reliability | Add concurrency, reconnect, timeout and failure-injection tests. | Defined behavior under engine failure and multiple simultaneous clients. |
| P2 — Scaling | Move rate limiting to shared storage and review session state for multiple workers. | Consistent limits and predictable behavior across replicas. |
| P2 — UX | Improve uncertainty presentation, model status and privacy notices. | Users understand categories are estimates and know when capture is active. |

## 9.2 Conclusion

MoodLens is a coherent full-stack prototype: it combines browser capture, REST and WebSocket APIs, two interchangeable analysis engines, live face tracking, dashboard visualization and container packaging. The module boundaries are appropriate for a project of this size, and the included tests provide a useful functional baseline.

The largest gap is not missing interface functionality; it is evidence and hardening. Before calling the project production-ready, the team should add resource limits and access controls, make failures observable, test concurrent usage, and evaluate model behavior on suitable labeled data. The application should continue to describe outputs as facial-expression estimates rather than definitive emotions.

## Appendix A. Repository map

| Path | Role |
| --- | --- |
| backend/app/main.py | FastAPI routes, image decode/inference orchestration and WebSocket stream. |
| backend/app/engines.py | Haar and optional DeepFace engine implementations and registry. |
| backend/app/tracker.py | IoU-based face association, smoothing and track expiry. |
| backend/app/config.py | Environment configuration and mood metadata. |
| backend/app/rate_limit.py | In-memory sliding-window rate limiter. |
| frontend/index.html / style.css / app.js | Dashboard structure, appearance and browser-side behavior. |
| tests/test_api.py | Functional API, WebSocket and tracker tests. |
| Dockerfile / docker-compose.yml | Container build and run configuration. |
| README.md | Features, setup, endpoints, configuration and deployment notes. |

## Appendix B. Review basis

This report is based on the supplied `MoodLens-main.zip` repository, including README documentation, backend modules, frontend assets, requirements, Docker configuration and the visible Pytest suite. No formal model benchmark, production traffic test, security audit or independent fairness study is claimed by this report.